SPI with Single Port RAM Verification Project

| Group number |  |
| --- | --- |
| 3 | كريم أيمن محمود يحيى |
| 2 | كريم حسن عاطف |
| 2 | Ahmed Mohamed Abdelmoneam |

Spi Slave part :

- Spi slave UVM architecture

Comment : first top set the virtual interface in configuration data base ,then test get this virtual interface and set  configuration data base with all interface  then agent get this configuration object and use it to connect between driver and monitor with the interface of DUT , Driver ask sequencer if there is data from sequence item , if there is data take data and give it to interface then monitor get data from interface and give it to scoreboard to compare it with refrence model by using fifo to get sequence item and give it to scoreboard , the same way for collector get sequence item and cover if sequence done , then after the simulation done test raise object and end simulation .

- Design file

module SLAVE (spi_slave_if.DUT ss_if);

localparam IDLE      = 3'b000;

localparam WRITE     = 3'b001;

localparam CHK_CMD   = 3'b010;

localparam READ_ADD  = 3'b011;

localparam READ_DATA = 3'b100;

reg [3:0] counter;

reg       received_address;

reg [2:0] cs, ns;

always @(posedge ss_if.clk) begin

if (~ss_if.rst_n) begin

cs <= IDLE;

end

else begin

cs <= ns;

end

end

always @(*) begin

case (cs)

IDLE : begin

if (ss_if.SS_n)

ns = IDLE;

else

ns = CHK_CMD;

end

CHK_CMD : begin

if (ss_if.SS_n)

ns = IDLE;

else begin

if (~ss_if.MOSI)

ns = WRITE;

else begin

if (received_address)

ns = READ_DATA;

else

ns = READ_ADD;

end

end

end

WRITE : begin

if (ss_if.SS_n)

ns = IDLE;

else

ns = WRITE;

end

READ_ADD : begin

if (ss_if.SS_n)

ns = IDLE;

else

ns = READ_ADD;

end

READ_DATA : begin

if (ss_if.SS_n)

ns = IDLE;

else

ns = READ_DATA;

end

endcase

end

always @(posedge ss_if.clk) begin

if (~ss_if.rst_n) begin

ss_if.rx_data <= 0;

ss_if.rx_valid <= 0;

received_address <= 0;

ss_if.MISO <= 0;

counter<=0; // reset

end

else begin

case (cs)

IDLE : begin

ss_if.rx_valid <= 0;

end

CHK_CMD : begin

counter <= 10;

end

WRITE : begin

if (counter > 0) begin

ss_if.rx_data[counter-1] <= ss_if.MOSI;

counter <= counter - 1;

end

else begin

ss_if.rx_valid <= 1;

end

end

READ_ADD : begin

if (counter > 0) begin

ss_if.rx_data[counter-1] <= ss_if.MOSI;

counter <= counter - 1;

end

else begin

ss_if.rx_valid <= 1;

received_address <= 1;

end

end

READ_DATA : begin

if (ss_if.tx_valid) begin

if (counter > 0) begin

ss_if.MISO <= ss_if.tx_data[counter-1];

counter <= counter - 1;

end

else begin

received_address <= 0;

ss_if.rx_valid <= 0;

end

end

else begin

if (counter > 0) begin // write data before going to read data

ss_if.rx_data[counter-1] <= ss_if.MOSI;

counter <= counter - 1;

end

else begin

ss_if.rx_valid <= 1;

counter <= 8;

end

end

end

endcase

end

end

// READ_ADD and READ_DATA is replaced

// counter not equal 0 at reset

`ifdef SIM

sequence write_add_seq;

(ss_if.SS_n==1) ##1 (ss_if.SS_n==0) ##1 (ss_if.MOSI == 0)[*3];

endsequence

sequence write_data_seq;

(ss_if.SS_n==1) ##1 (ss_if.SS_n==0) ##1 (ss_if.MOSI == 0)[*2] ##1(ss_if.MOSI==1);

endsequence

sequence read_add_seq;

(ss_if.SS_n==1) ##1 (ss_if.SS_n==0) ##1 (ss_if.MOSI == 1)[*2] ##1(ss_if.MOSI==0);

endsequence

sequence read_data_seq;

(ss_if.SS_n==1) ##1 (ss_if.SS_n==0) ##1 (ss_if.MOSI == 1)[*3];

endsequence

property chck_rx_valid;

@(posedge ss_if.clk) disable iff(~ss_if.rst_n)

(write_add_seq or write_data_seq or read_add_seq or read_data_seq) |=> ##9 ($rose(ss_if.rx_valid) && $rose(ss_if.SS_n)[->1]);

endproperty

property chck_reset ;

@(posedge ss_if.clk)   (~ss_if.rst_n) |=>(ss_if.MISO ==0 && ss_if.rx_valid==0 && ss_if.rx_data==0);

endproperty

property chck_state_idle;

@(posedge ss_if.clk) disable iff(~ss_if.rst_n) (cs==IDLE && !ss_if.SS_n) |=>(cs==CHK_CMD);

endproperty

property chck_state_write;

@(posedge ss_if.clk) disable iff(~ss_if.rst_n) (cs==CHK_CMD && !ss_if.SS_n && !ss_if.MOSI) |=>  (cs==WRITE);

endproperty

property chck_state_read_add;

@(posedge ss_if.clk) disable iff(~ss_if.rst_n)  (cs==CHK_CMD && !ss_if.SS_n && ss_if.MOSI && !received_address) |=>(cs==READ_ADD);

endproperty

property chck_state_read_data;

@(posedge ss_if.clk) disable iff(~ss_if.rst_n)  (cs==CHK_CMD && !ss_if.SS_n && ss_if.MOSI && received_address) |=>(cs==READ_DATA);

endproperty

property chck_state_write_to_idle;

@(posedge ss_if.clk)  (cs==WRITE &&(~ss_if.rst_n)) |=> (cs==IDLE);

endproperty

property chck_state_read_add_to_idle;

@(posedge ss_if.clk)  (cs==READ_ADD && (~ss_if.rst_n)) |=> (cs==IDLE);

endproperty

property chck_state_read_datato_idle;

@(posedge ss_if.clk)  (cs==READ_DATA && (~ss_if.rst_n)) |=> (cs==IDLE);

endproperty

assert property (chck_reset);

assert property (chck_rx_valid);

assert property (chck_state_idle);

assert property (chck_state_write);

assert property (chck_state_read_add);

assert property (chck_state_read_data);

assert property (chck_state_write_to_idle);

assert property (chck_state_read_add_to_idle);

assert property (chck_state_read_datato_idle);

cover property (chck_reset);

cover property (chck_rx_valid);

cover property (chck_state_idle);

cover property (chck_state_write);

cover property (chck_state_read_add);

cover property (chck_state_read_data);

cover property (chck_state_write_to_idle);

cover property (chck_state_read_add_to_idle);

cover property (chck_state_read_datato_idle);

`endif

Endmodule

- Design Bugs

Figure 1 bug 1

Figure 2 bug1 fixed

Figure 3 bug 2

Figure 4 bug2 fixed

Figure 5 bug 3

Figure 6 bug 3 fixed

Figure 7 bug 4

Figure 8 bug4 fixed

- Refrence model file

module SPI_Slave(MOSI,SS_n,clk,rst_n,rx_data,tx_valid,tx_data,MISO,rx_valid);

input MOSI,SS_n,clk,rst_n,tx_valid;

input [7:0] tx_data;

output reg MISO,rx_valid;

output  reg [9:0] rx_data;

parameter IDLE=3'b000;

parameter CHK_CMD=3'b001;

parameter WRITE=3'b010;

parameter READ_ADD=3'b011;

parameter  READ_DATA=3'b100;

reg ADDRESS_read; // this is signal to increment when  reading an address

reg[3:0] counter;

reg [2:0] cs ,ns;

always@(posedge clk) begin

if(~rst_n)

cs<=IDLE;

else

cs<=ns;

end

always@(*) begin

case(cs)

IDLE: begin

if(SS_n)

ns=IDLE;

else

ns=CHK_CMD;

end

CHK_CMD : begin

if (SS_n)

ns = IDLE;

else begin

if( ~MOSI) begin

ns=WRITE;

end

else begin

casex(ADDRESS_read)

1'b0 : ns=READ_ADD;

1'b1: ns=READ_DATA;

1'bx: ns=READ_ADD;

endcase

end

end

end

WRITE: begin

if(SS_n==0 )

ns=WRITE;

else begin

ns=IDLE;

end

end

READ_ADD :  begin

if(SS_n==0) begin

ns=READ_ADD;

end

else begin

ns=IDLE;

end

end

READ_DATA : begin

if(SS_n==0)

ns=READ_DATA;

else begin

ns=IDLE;

end

end

endcase

end

always @(posedge clk) begin

if (~rst_n) begin

rx_data <= 0;

rx_valid <= 0;

ADDRESS_read<= 0;

MISO <= 0;

end

else begin

case(cs)

IDLE: rx_valid<=0;

CHK_CMD : begin

counter<=10;

end

WRITE  : begin

if(counter>0) begin

rx_data[counter-1]<=MOSI;

counter<=counter-1;

end

else begin

rx_valid<=1;

end

end

READ_ADD : begin

if(counter>0) begin

rx_data[counter-1]<=MOSI;

counter<=counter-1;

end

else begin

rx_valid<=1;

ADDRESS_read<=1;

end

end

READ_DATA : begin

if (tx_valid) begin

if(counter>0) begin

MISO<=tx_data[counter-1];

counter<=counter-1;

end

else begin

ADDRESS_read <= 0;

rx_valid<=0;

end

end

else begin

if(counter>0) begin

rx_data[counter-1]<=MOSI;

counter<=counter-1;

end

else begin

rx_valid<=1;

counter<=9;

end

end

end

endcase

end

end

endmodule

- Interface file

interface spi_slave_if (clk);

input  bit clk;

logic           MOSI, rst_n, SS_n, tx_valid;

logic      [7:0] tx_data;

logic [9:0] rx_data,rx_data_ref;

logic       rx_valid, MISO,MISO_ref , rx_valid_ref;

modport DUT(input clk,MOSI,rst_n,SS_n,tx_data,tx_valid, output MISO,rx_data,rx_valid);

endinterface : spi_slave_if

- Top file

import uvm_pkg::*;

`include "uvm_macros.svh"

import spi_slave_test_pkg::*;

import spi_slave_seq_item_package::*;

import shared_pkg::*;

module top();

bit clk;

initial begin

forever #2 clk=~clk;

end

spi_slave_if ss_if(clk);

SLAVE DUT(ss_if);

SPI_Slave dut(ss_if.MOSI,ss_if.SS_n,ss_if.clk,ss_if.rst_n,ss_if.rx_data_ref,ss_if.tx_valid,ss_if.tx_data,ss_if.MISO_ref,ss_if.rx_valid_ref);

initial begin

uvm_config_db #(virtual spi_slave_if) :: set(null,"uvm_test_top","SPI_IF",ss_if);

run_test ("spi_slave_test");

end

endmodule

- Sequence item file

package spi_slave_seq_item_package;

import uvm_pkg::*;

`include "uvm_macros.svh"

import shared_pkg::*;

class spi_slave_seq_item extends uvm_sequence_item;

`uvm_object_utils(spi_slave_seq_item);

rand logic [7:0] tx_data;

logic [9:0] rx_data ,rx_data_ref;

logic rx_valid, MISO,MISO_ref ,rx_valid_ref;

logic MOSI;

rand logic SS_n;

rand logic rst_n;

rand logic tx_valid;

rand bit[0:10] array_rand;

function new(string name= "spi_slave_seq_item");

super.new(name);

endfunction

//reset

constraint reset

{

rst_n  dist { 1:/98 , 0:/2};

}

//SS_n for all cases

constraint serial_comm_all_cases

{

if(  array_rand[0:2] inside {3'b000, 3'b001, 3'b110} && counter_allcases%14 !=0) {

SS_n==0;

}

else

{

SS_n==1;

}

}

//SS_n for read data

constraint serial_comm_read_data{

if(  array_rand[0:2] inside {3'b111} && counter_read%24 !=0) {

SS_n==0;

}

else

{

SS_n==1;

}

}

//tx_valid for read data

constraint trans_ram

{

if (array_rand[0:2] == 3'b111  && counter_read==23)

{

tx_valid==1;

}

if(array_rand[0:2] != 3'b111)

{

tx_valid==0;

}

}

// array_rand for all cases

constraint mosi_in

{

if (SS_n_prev && !SS_n)

array_rand[0:2] inside {3'b000, 3'b001, 3'b110 , 3'b111};

}

function void post_randomize;

SS_n_prev=SS_n;

if(counter_allcases<11 )

MOSI=array_rand[counter_allcases];

endfunction

function update_counter_allcases;

counter_allcases++;

if (counter_allcases==14 )

counter_allcases=0;

endfunction

function update_counter_read;

counter_read++;

if (counter_read==24 )

counter_read=0;

endfunction

function string convert2string();

return $sformatf("%s reset = 0b%0b , mosi=%b , miso=%b , ss_n = %b ",super.convert2string(),rst_n,MOSI,MISO,SS_n);

endfunction

function string convert2string_stimulus();

return $sformatf("reset = 0b%0b , mosi=%b , ss_n = %b ",rst_n,MOSI,SS_n);

endfunction

endclass

endpackage

- Main sequence file

package spi_slave_main_sequence_package;

import uvm_pkg::*;

`include "uvm_macros.svh"

import spi_slave_seq_item_package::*;

import shared_pkg::*;

class spi_slave_main_sequence extends  uvm_sequence #(spi_slave_seq_item);

`uvm_object_utils(spi_slave_main_sequence);

spi_slave_seq_item seq_item_main;

function new(string name= "spi_slave_main_sequence");

super.new(name);

endfunction

task body ;

seq_item_main= spi_slave_seq_item :: type_id :: create("seq_item_main");

repeat(1000) begin

start_item(seq_item_main);

seq_item_main.reset.constraint_mode(1);

seq_item_main.serial_comm_all_cases.constraint_mode(1);

seq_item_main.mosi_in.constraint_mode(1);

seq_item_main.serial_comm_read_data.constraint_mode(0);

seq_item_main.trans_ram.constraint_mode(0);

if(counter_allcases==0) begin

seq_item_main.array_rand.rand_mode(1);

$display("array_rand=%b , time =%t ,conter=%d",seq_item_main.array_rand,$time,counter_allcases);

end

else begin

seq_item_main.array_rand.rand_mode(0);

end

assert (seq_item_main.randomize() with {seq_item_main.array_rand[0:2] inside {3'b000,3'b001,3'b110};} );

seq_item_main.update_counter_allcases;

finish_item(seq_item_main);

end

repeat(1000) begin

start_item(seq_item_main);

seq_item_main.reset.constraint_mode(1);

seq_item_main.serial_comm_all_cases.constraint_mode(0);

seq_item_main.mosi_in.constraint_mode(0);

seq_item_main.serial_comm_read_data.constraint_mode(1);

seq_item_main.trans_ram.constraint_mode(1);

if(counter_read==0) begin

seq_item_main.array_rand.rand_mode(1);

$display("array_rand=%b , time =%t ,conter=%d",seq_item_main.array_rand,$time,counter_read);

end

else begin

seq_item_main.array_rand.rand_mode(0);

end

assert (seq_item_main.randomize() with {seq_item_main.array_rand[0:2] inside {3'b111};} );

seq_item_main.update_counter_read;

finish_item(seq_item_main);

end

endtask

endclass

endpackage

- Reset sequence file

package spi_slave_reset_sequence_package;

import uvm_pkg::*;

`include "uvm_macros.svh"

import spi_slave_seq_item_package::*;

class spi_slave_reset_sequence extends  uvm_sequence #(spi_slave_seq_item);

`uvm_object_utils(spi_slave_reset_sequence);

spi_slave_seq_item seq_item;

function new(string name= "spi_slave_reset_sequence");

super.new(name);

endfunction

task body ;

seq_item= spi_slave_seq_item :: type_id :: create("seq_item");

start_item(seq_item);

seq_item.rst_n=1;

seq_item.rx_valid=0;

seq_item.tx_valid=0;

seq_item.SS_n=1;

finish_item(seq_item);

endtask

endclass

endpackage

- Sequencer file

package spi_slave_sequencer_package ;

import uvm_pkg::*;

`include "uvm_macros.svh"

import spi_slave_seq_item_package::*;

class spi_slave_sequencer extends uvm_sequencer #(spi_slave_seq_item);

`uvm_component_utils(spi_slave_sequencer)

function new(string name= "spi_slave_sequencer",uvm_component parent=null);

super.new(name,parent);

endfunction

endclass

endpackage

- Configuration object file

package spi_slave_config_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

class spi_slave_config extends uvm_object;

`uvm_object_utils(spi_slave_config) // factory store name of class

virtual spi_slave_if ss_vif; //  create interface inside configration class

function new(string name= "spi_slave_config");

super.new(name);

endfunction

endclass

endpackage

- Shared pkg

package shared_pkg;

int counter_allcases =0  ;

int counter_read=0;

logic SS_n_prev=1;

endpackage

- Driver

package spi_slave_driver_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

import spi_slave_seq_item_package::*;

import shared_pkg ::*;

class spi_slave_driver extends uvm_driver #(spi_slave_seq_item);

`uvm_component_utils(spi_slave_driver);

virtual spi_slave_if ss_vif;

spi_slave_seq_item seq_item;

function new(string name ="spi_slave_driver",uvm_component parent =null);

super.new(name,parent);

endfunction

task run_phase(uvm_phase phase);

super.run_phase(phase);

forever begin

seq_item =  spi_slave_seq_item :: type_id :: create ( "seq_item");

seq_item_port.get_next_item(seq_item);

ss_vif.rst_n=seq_item.rst_n;

ss_vif.SS_n=seq_item.SS_n;

ss_vif.MOSI=seq_item.MOSI;

ss_vif.tx_valid=seq_item.tx_valid;

ss_vif.tx_data=seq_item.tx_data;

@(negedge ss_vif.clk);

seq_item_port.item_done();

`uvm_info ("run_phase",seq_item.convert2string_stimulus(),UVM_HIGH)

end

endtask

endclass

endpackage

- Monitor

package spi_slave_monitor_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

import  spi_slave_seq_item_package::*;

import shared_pkg ::*;

class spi_slave_monitor extends uvm_monitor;

`uvm_component_utils(spi_slave_monitor);

virtual spi_slave_if ss_vif;

spi_slave_seq_item seq_item;

uvm_analysis_port #(spi_slave_seq_item) mon_ap;

function new(string name ="spi_slave_monitor",uvm_component parent =null);

super.new(name,parent);

endfunction

function void build_phase( uvm_phase phase);

super.build_phase(phase);

mon_ap = new("mon_ap", this);

endfunction

task run_phase (uvm_phase phase);

super.run_phase(phase);

forever begin

seq_item=spi_slave_seq_item::type_id::create("seq_item");

@(negedge ss_vif.clk);

seq_item.rst_n=ss_vif.rst_n;

seq_item.MOSI=ss_vif.MOSI;

seq_item.SS_n=ss_vif.SS_n;

seq_item.MISO=ss_vif.MISO;

seq_item.MISO_ref=ss_vif.MISO_ref;

seq_item.tx_valid=ss_vif.tx_valid;

seq_item.tx_data=ss_vif.tx_data;

seq_item.rx_valid=ss_vif.rx_valid;

seq_item.rx_data=ss_vif.rx_data;

seq_item.rx_valid_ref=ss_vif.rx_valid_ref;

seq_item.rx_data_ref=ss_vif.rx_data_ref;

mon_ap.write(seq_item);

`uvm_info ("run_phase",seq_item.convert2string_stimulus(),UVM_HIGH)

end

endtask

endclass

endpackage

- Agent

package spi_slave_agent_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

import  spi_slave_seq_item_package::*;

import spi_slave_config_pkg::*;

import  spi_slave_driver_pkg::*;

import spi_slave_monitor_pkg::*;

import spi_slave_sequencer_package::*;

class spi_slave_agent extends uvm_agent;

`uvm_component_utils(spi_slave_agent)

spi_slave_seq_item seq_item;

spi_slave_config           s_cfg;

spi_slave_driver       drv;

spi_slave_sequencer sqr;

spi_slave_monitor       mon;

uvm_analysis_port #(spi_slave_seq_item) agt_ap;

function new(string name = "spi_slave_agent", uvm_component parent = null);

super.new(name, parent);

endfunction

function void build_phase(uvm_phase phase);

super.build_phase(phase);

if (!uvm_config_db#(spi_slave_config)::get(this, "", "CFG", s_cfg)) begin

`uvm_fatal("build_phase", "unable to get configuration object")

end

sqr = spi_slave_sequencer::type_id::create("sqr", this);

drv = spi_slave_driver::type_id::create("drv", this);

mon = spi_slave_monitor::type_id::create("mon", this);

agt_ap = new("agt_tb",this);

endfunction

function void connect_phase(uvm_phase phase);

super.connect_phase(phase);

drv.ss_vif = s_cfg.ss_vif;

mon.ss_vif = s_cfg.ss_vif;

drv.seq_item_port.connect(sqr.seq_item_export);

mon.mon_ap.connect(agt_ap);

endfunction

endclass

endpackage

16. Scoreboard

package spi_slave_scoreboard_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

import  spi_slave_seq_item_package::*;

class spi_slave_scoreboard extends uvm_scoreboard ;

`uvm_component_utils(spi_slave_scoreboard);

uvm_analysis_export #(spi_slave_seq_item) sb_export;

uvm_tlm_analysis_fifo  #(spi_slave_seq_item) sb_fifo;

spi_slave_seq_item sq_item_sb;

int err_count=0;

int correct_count=0;

function new(string name ="spi_slave_scoreboard",uvm_component parent =null);

super.new(name,parent);

endfunction

function void build_phase (uvm_phase phase);

super.build_phase(phase);

sb_export = new("sb_export",this);

sb_fifo = new("sb_fifo" , this);

endfunction

function void connect_phase (uvm_phase phase);

super.connect_phase(phase);

sb_export.connect(sb_fifo.analysis_export);

endfunction

task run_phase (uvm_phase phase);

super.run_phase(phase);

forever begin

sb_fifo.get(sq_item_sb);

if (sq_item_sb.MISO != sq_item_sb.MISO_ref || sq_item_sb.rx_valid !=sq_item_sb.rx_valid_ref || sq_item_sb.rx_data!=sq_item_sb.rx_data_ref) begin

`uvm_error("run_phase",$sformatf("Comparison failed, Transaction received by the DUT: %s While the reference out:0b%0b",

sq_item_sb.convert2string(),sq_item_sb.MISO_ref))

err_count++;

end

else begin

`uvm_info("run_phase",$sformatf("Correct ALU out: %s", sq_item_sb.convert2string()), UVM_HIGH)

correct_count++;

end

end

endtask

function void report_phase(uvm_phase phase) ;

super. report_phase (phase) ;

`uvm_info("report_phase", $sformatf("Total successful transactions: %0d", correct_count), UVM_MEDIUM);

`uvm_info("report_phase", $sformatf("Total failed transactions: %0d", err_count), UVM_MEDIUM);

endfunction

endclass

endpackage

17. collector

package  spi_slave_collector_package ;

import uvm_pkg::*;

`include "uvm_macros.svh"

import  spi_slave_seq_item_package::*;

import shared_pkg::*;

class spi_slave_coverage extends uvm_component;

`uvm_component_utils(spi_slave_coverage)

uvm_analysis_export #(spi_slave_seq_item) cov_export;

uvm_tlm_analysis_fifo #(spi_slave_seq_item) cov_fifo;

spi_slave_seq_item seq_item_cov;

covergroup covcode ;

reciver_data: coverpoint seq_item_cov.rx_data[9:8];

ss_n_allcases : coverpoint seq_item_cov.SS_n

{

bins trans_all_cases= (1 => 0[*13] =>1) ;

}

ss_n_read_data : coverpoint seq_item_cov.SS_n

{

bins trans_read= (1 => 0[*23] =>1) ;

}

ss_n : coverpoint seq_item_cov.SS_n

{

bins ss_n_val={0};

}

mosi: coverpoint seq_item_cov.MOSI

{

bins write_add = (0=>0=>0);

bins write_data = (0=>0=>1);

bins read_add = (1=>1=>0);

bins read_data =(1=>1=>1);

bins mosi_val_low={0};

bins mosi_val_high={1};

}

cross_mosi_ss_n : cross mosi,ss_n

{

option.cross_auto_bin_max=0;

bins ss_n_low_mosi_low = binsof(ss_n.ss_n_val) && binsof(mosi.mosi_val_low);

bins ss_n_low_mosi_high = binsof(ss_n.ss_n_val)  && binsof(mosi.mosi_val_high);

}

endgroup

function new(string name = "spi_slave_coverage", uvm_component parent = null);

super.new(name, parent);

covcode=new();

endfunction

function void build_phase(uvm_phase phase);

super.build_phase(phase);

cov_export = new("cov_export", this);

cov_fifo   = new("cov_fifo", this);

endfunction

function void connect_phase(uvm_phase phase) ;

super.connect_phase(phase);

cov_export.connect(cov_fifo.analysis_export);

endfunction

task run_phase(uvm_phase phase);

super.run_phase(phase);

forever begin

cov_fifo.get(seq_item_cov);

covcode.sample();

end

endtask

endclass

endpackage

18. Environment

package spi_slave_env_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

import spi_slave_agent_pkg::*;

import spi_slave_scoreboard_pkg::*;

import spi_slave_collector_package::*;

class spi_slave_env extends uvm_env;

`uvm_component_utils(spi_slave_env);

spi_slave_scoreboard score;

spi_slave_coverage cover_grp;

spi_slave_agent agt;

function new(string name ="spi_slave_env",uvm_component parent =null);

super.new(name,parent);

endfunction

function void build_phase (uvm_phase phase);

super.build_phase(phase);

agt = spi_slave_agent :: type_id ::create("agt",this);

score =   spi_slave_scoreboard :: type_id :: create("score",this);

cover_grp = spi_slave_coverage  ::type_id ::create("cover_grp",this);

endfunction

function void connect_phase (uvm_phase phase);

agt.agt_ap.connect(score.sb_export);

agt.agt_ap.connect(cover_grp.cov_export);

endfunction

endclass

endpackage

19. Test

package spi_slave_test_pkg;

import uvm_pkg::*;

`include "uvm_macros.svh"

import spi_slave_env_pkg::*;

import spi_slave_reset_sequence_package::*;

import spi_slave_main_sequence_package::*;

import spi_slave_config_pkg ::*;

class spi_slave_test extends uvm_test;

`uvm_component_utils(spi_slave_test);

spi_slave_env  env;

spi_slave_config   s_cfg;

spi_slave_reset_sequence rst_seq;

spi_slave_main_sequence main_seq;

function new(string name ="spi_slave_test",uvm_component parent =null);

super.new(name,parent);

endfunction

function void build_phase (uvm_phase phase);

super.build_phase(phase);

env =spi_slave_env :: type_id :: create("env",this);

s_cfg = spi_slave_config   :: type_id :: create ("s_cfg");

rst_seq= spi_slave_reset_sequence :: type_id :: create ("rst_seq");

main_seq=spi_slave_main_sequence :: type_id :: create("main_seq");

if(!uvm_config_db #(virtual spi_slave_if) :: get(this,"","SPI_IF",s_cfg.ss_vif))

`uvm_fatal("build_phase","Test - unable to get the virtual interface");

uvm_config_db #(spi_slave_config) :: set(this,"*","CFG",s_cfg);

endfunction

task run_phase(uvm_phase phase);

super.run_phase(phase);

phase.raise_objection(this);

`uvm_info("run_phase","reset_asserted",UVM_LOW)

rst_seq.start(env.agt.sqr);

`uvm_info("run_phase","reset_deasserted",UVM_LOW)

`uvm_info ("run_phase","stimulus generation begin",UVM_LOW)

main_seq.start(env.agt.sqr);

`uvm_info ("run_phase","stimulus generation done",UVM_LOW)

phase.drop_objection(this);

endtask

endclass: spi_slave_test

endpackage

20. Src_file

21. Do file

22. Functional coverage

23. Code coverage

24. Assertion coverage

25. Questasim snippets

Figure 9 when ss_n is deasserted mosi read_add sequence (110)

Figure 10 when ss_n is deasserted mosi read_data sequence (111)

Figure 11 when ss_n is deasserted mosi write_add sequence (000)

Figure 12 when ss_n is deasserted mosi write_data sequence (001)

26. Table of assertions with features

| Feature | Assertion |
| --- | --- |
| Whenever the rst is asserted, MISO , rx_valid , rx_data is low | @(posedge ss_if.clk) (~ss_if.rst_n) ¦=>(ss_if.MISO ==0 && ss_if.rx_valid==0 && ss_if.rx_data==0); |
| Whenever the current state is idle and SS_n is low so current state will be CHCK_CMD in next cycle and reset is high | @(posedge ss_if.clk) disable iff(~ss_if.rst_n) (cs==IDLE && !ss_if.SS_n) ¦=>(cs==CHK_CMD); |
| Whenever the current state CHCK_CMD and SS_n is low and mosi is low so current state will be WRITE in next cycle and reset is high | @(posedge ss_if.clk) disable iff(~ss_if.rst_n) (cs==CHK_CMD && !ss_if.SS_n && !ss_if.MOSI) ¦=> (cs==WRITE); |
| Whenever the current state CHCK_CMD and SS_n is low and mosi is high and received_add is low so current state will be WRITE in next cycle and reset is high | @(posedge ss_if.clk) disable iff(~ss_if.rst_n) (cs==CHK_CMD && !ss_if.SS_n && ss_if.MOSI && !received_address) ¦=>(cs==READ_ADD); |
| Whenever the current state CHCK_CMD and SS_n is low and mosi is high and received_add is high so current state will be WRITE in next cycle and reset is high | @(posedge ss_if.clk) disable iff(~ss_if.rst_n) (cs==CHK_CMD && !ss_if.SS_n && ss_if.MOSI && received_address) ¦=>(cs==READ_DATA); |
| Whenever the rst is asserted and current state is WRITE , the current state will be IDEAL in next clock cycle | @(posedge ss_if.clk) (cs==WRITE &&(~ss_if.rst_n)) ¦=> (cs==IDLE); |
| Whenever the rst is asserted and current state is READ_ADD , the current state will be IDEAL in next clock cycle | @(posedge ss_if.clk) (cs==READ_ADD && (~ss_if.rst_n)) ¦=> (cs==IDLE); |
| Whenever the rst is asserted and current state is READ_DATA , the current state will be IDEAL in next clock cycle | @(posedge ss_if.clk) (cs==READ_DATA && (~ss_if.rst_n)) ¦=> (cs==IDLE); |
| Whenever SS_n is deasserted and wite_add or write_data or read_add or read_data sequence done then rx_valid will assert and SS_n will assert to end communication | @(posedge ss_if.clk) disable iff(~ss_if.rst_n) (write_add_seq or write_data_seq or read_add_seq or read_data_seq) ¦=> ##9 ($rose(ss_if.rx_valid) && $rose (ss_if.SS_n)[->1]); |

Sequence which I mentioned in last row :

RAM Part:

| Feature | Assertion |
| --- | --- |
| When rst_n is asserted, dout & tx_valid are low | (@(posedge clk) (!rst_n) ¦=> (dout == 0) && (tx_valid == 0)) |
| When din[9:8] = 11 and rx_valid is asserted, tx_valid should be high and should eventually fall. | @(posedge clk) disable iff(!rst_n) ((din[8] && din[9]) && rx_valid) ¦=> (tx_valid == 1) ¦=> $fell(tx_valid)[->1]) |
| When din[9:8] = 10, tx_valid should be low. | (@(posedge clk) disable iff(!rst_n) (!(din[8] && din[9])) ¦=> (tx_valid == 0)) |
| When din[9:8] = 00 which is a write address instruction, there should eventually be a write data instruction din[9:8] = 01 | (@(posedge clk) disable iff(!rst_n) ((!din[8] && !din[9])) ¦=> (din[8] && !din[9])[->1]); |
| When din[9:8] = 10 which is a read address instruction, there should eventually be a read data instruction din[9:8] = 11 | (@(posedge clk) disable iff(!rst_n) ((!din[8] && din[9])) ¦=> (din[8] && din[9])[->1]); |

Ram Uvm architecture :

Top:

Test:

Env:

Agent:

Config Object, Sequence Item and Sequencer:

Reset, Write only sequence, Read only sequence and W&R sequence:

Monitor and Driver:

Interface:

Coverage:

Scoreboard:

Reference Model:

Design:

Bugs Found:

- In READ_DATA case, output would read from Wr_addr.

- Missing begin end in else branch

Shared_Pkg:

Do File:

Assertions:

Waveforms:

Reset Sequence:

Write-Only Sequence:

Read-Only Sequence:

Write & Read Sequence:

Transcript:

Functional Coverage:

Assertions Coverage:

Code Coverage:

Default branch excluded as all cases are covered.

Wrapper part :